Attachment № 1 to the invitation

TERMS OF REFERENCE

for participation in a procurement procedure with subject

„Upgrade of the Airfield Ground Lighting Control and Monitoring System (ALCMS) of the Airfield Ground Lighting System (AGL) at Sofia Airport”

- PURPOSE

This ToR is prepared in connection with the need to increase flight safety, as well as in connection with the certification of SOF Connect AD as an organization approved to maintain the AGL System at Sofia Airport.

- REQUIREMENTS

Sofia Airport AGL System was installed in the period 2005 – 2007. During the installation, devices, materials and software manufactured and supplied by the Swedish company – SAFEGATE were used. Spare parts from these manufacturers have been used in the operation and repair of the AGL System so far.

The products and software listed below must be compatible and meet the technical parameters of those currently installed.

| № | NAME |
| --- | --- |
| - ALCMS core update |  |
| 1. | Upgrade of ALCMS software in Sofia allowing future updates to be more efficient and less expensive, includes: - ALCMS composer configuration base update; - ALCMS application upgrade; - Possibility to modify stopbar timeouts by the airport personal. |
| 2. | Delivery, installation and commissioning of new NCUs and network switches and DC power equipment for existing cabinets in TP2, 3 and 4, as follows: - 2 pcs. NCU in TP2; - 1 pcs. NCU in TP3; - 1 pcs. NCU in TP4; - DC powers and network switches in all three substations; - In-house engineering and commissioning. |
| 3. | Redundant NCUs for TP3 and TP4, includes: - 1 pcs. NCU in TP3; - 1 pcs. NCU in TP4; - New MOXA cards between NCUs and SCM, MOXA cables and distribution boards etc. for redundant connection; - In-house engineering and commissioning on site at the same time with other NCUs. |
| 4. | Replacement of existing PCs, includes: - 1 pcs. in Tower VCR; - 1 pcs. in Tower GMC; - 1 pcs. Maintenace computer; - 2 pcs. in AGL servers; - In-house engineering and commissioning. |
| 5. | Delivery and installation, programminng and commissioning of SCM Board complete. |
| 6. | ALCMS update with Intensity progressive start of the lights. |
| - CCR interface change from Profibus to Modbus |  |
| 1. | CCR interface change to Modbus in ALCMS, includes: - New network switches for ALCMS cabinets in substations TP2, TP3 and TP4 for connecting CCRs with Modbus interface cards; - Inhouse engineering for SW changes.Commissioning on site. |
| 2. | IDM 8000 SERIAL, MODBUS TCP. |
| 3. | CCR Firmware update on site for all IDM 8000 CCRs where applicable for Modbus interface. |
| - ALCMS other updates |  |
| 1. | Sofia ALCMS update for adding Generator and UPS interface, includes: - Add the interface to the Generators and UPS in Sofia’s substation TP2, TP3 and TP4; - On-site assistance with testing of the Generator and UPS interface; - Animation of the Generator and UPS interface; - Commissioning on site. |
| 2. | New maintenance PC to TP2 including monitor and including commissioning on site. |
| - ALCMS/ILCMS check on site |  |
| 1. | ILCMS check on site. |
| 2. | Investigation of ALCMS alarms on site with reporting. |
| 3. | Inspection of each ILCMS circuit with reporting and spare parts recommendations. |
| 4. | Corrective actions on the ILCMS system. |
| 5. | Some spares will be brought and charged as per usage. |
| - New Runway Guard Lights for TWYs “A”, “B’, “C”, “D”, “F”, “E” |  |
| 1. | Delivery and installation of new LED Runway Guard lights SL-IQ1-RGL-E, includes: - LED Runway Guard lights SL-IQ1-RGL-E; - Trench and backfilling, cutting pavement for cables, installation of secondary cable, sealing, anchoring, mounting and testing of RGL lights; - All necessary materials and accessories like: secondary cables and connector kits, polyurethane- joint sealant and rope, anchors, concrete foundations, base plates, breakable couplings. |
| - ILCMS updates |  |
| 1. | Update of ALCMS/ILCMS software for adding 8 pairs of RGL lights to existing ILCMS circuits. |
| 2. | Commissioning on site. |
| 3. | AMT Basic - Latest AMT (ASP Maintenance Tool) software version. |
| - MBD equipment for TWYs “A”, “B’, “C”, “D”, “F” |  |
| 1. | Delivery and commissioning, including: - One MBD pair, new model 316: transmitter and receiver; - 60mm tube Length 1.10m; - EL-FIX BSP/Coupling_60MM_Assy; - IDM 5838 BASE PLATE; - SIU Sensor Interface Unit; - Installation works, include: trenches, foundations, cables and installation of MBD and accessories; - Commissioning and testing of MBD circuits on site. |
| 2. | ALCMS engineer remote support during commission. |

- OTHER REQUIREMENTS

  - Deadline for delivery: delivery within 150 (one hundred and fifty) calendar days from signing the contract.

  - Delivery shall take place on the territory of Sofia Airport.

  - Method of payment: 100% after execution or in advance, against a 100% bank guarantee.

  - Validity of the offer – not less than 30 calendar days from the date of receipt of the offer.

  - Warranty period for products and software – not less than 24 (twenty four) months.

  - All products must have quality certificates from the manufacturer, and the same shall be provided to the user.

  - All products supplied must be new and non-recycled.